Thrust Directions Table ([M0], [M1], [M2], [M3])

[M3]↗                    ↖[M0]

\                      /

\              /

\       /

[ROV]

/       \

/              \

/                    \

[M2]↘                            ↙[M1]

| Motion | M3 (↗) | M2 (↘) | M1 (↙) | M0 (↖) |
| --- | --- | --- | --- | --- |
| Forward | +1 | -1 | -1 | +1 |
| Backward | -1 | +1 | +1 | -1 |
| Right | +1 | +1 | -1 | -1 |
| Left | -1 | -1 | +1 | +1 |
| Forward-Right | +1 | 0 | -1 | 0 |
| Forward-Left | 0 | -1 | 0 | +1 |
| Backward-Right | 0 | +1 | 0 | -1 |
| Backward-Left | -1 | 0 | +1 | 0 |
| Rotate-Right | +1 | -1 | +1 | -1 |
| Rotate-Left | -1 | +1 | -1 | +1 |

M[0] (Front-Right, 135° Tilt)   →   Thrusts Up-Left (↖)

M[1] (Back-Right, 225° Tilt)    →   Thrusts Down-Left (↙)

M[2] (Back-Left, -45° Tilt)       →   Thrusts Down-Right (↘)

M[3] (Front-Left, 45° Tilt)       →   Thrusts Up-Right (↗)

Movement  Directions Table

- joystick inputs (no interference)

- button[ ] pressed to R-R, R-L.

| Motion | X-Axis | Y-Axis | R-Z | X-Axis + 1 | Y-Axis+1 | R-Z + 1 | Index |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward | 0 | -1 | 0 | 1 | 0 | 1 | 0 |
| Backward | 0 | 1 | 0 | 1 | 2 | 1 | 1 |
| Right | 1 | 0 | 0 | 2 | 1 | 1 | 2 |
| Left | -1 | 0 | 0 | 0 | 1 | 1 | 3 |
| Forward-Right | 1 | -1 | 0 | 2 | 0 | 1 | 4 |
| Forward-Left | -1 | -1 | 0 | 0 | 0 | 1 | 5 |
| Backward-Right | 1 | 1 | 0 | 2 | 2 | 1 | 6 |
| Backward-Left | -1 | 1 | 0 | 0 | 2 | 1 | 7 |
| Rotate-Right | 0 | 0 | 1 | 1 | 1 | 2 | 8 |
| Rotate-Left | 0 | 0 | -1 | 1 | 1 | 0 | 9 |

- joystick inputs (interference with R-Z)

- no button[ ] pressed to R-R, R-L.

| Motion | X-Axis | Y-Axis | R-Z | X-Axis + 1 | Y-Axis+1 | R-Z + 1 | Index |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward | 0 | -1 | 1 | 1 | 0 | 2 | 0 |
| Backward | 0 | 1 | 1 | 1 | 2 | 2 | 1 |
| Right | 1 | 0 | 1 | 2 | 1 | 2 | 2 |
| Left | -1 | 0 | 1 | 0 | 1 | 2 | 3 |
| Forward | 0 | -1 | -1 | 1 | 0 | 0 | 0 |
| Backward | 0 | 1 | -1 | 1 | 2 | 0 | 1 |
| Right | 1 | 0 | -1 | 2 | 1 | 0 | 2 |
| Left | -1 | 0 | -1 | 0 | 1 | 0 | 3 |

- joystick inputs (interference with R-Z)

- no button[ ] pressed to R-R, R-L.

| Motion | X-Axis | Y-Axis | R-Z | X-Axis + 1 | Y-Axis+1 | R-Z + 1 | Index |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward-Right | 1 | -1 | 1 | 2 | 0 | 2 | 4 |
| Forward-Left | -1 | -1 | 1 | 0 | 0 | 2 | 5 |
| Backward-Right | 1 | 1 | 1 | 2 | 2 | 2 | 6 |
| Backward-Left | -1 | 1 | 1 | 0 | 2 | 2 | 7 |
| Forward-Right | 1 | -1 | -1 | 2 | 0 | 0 | 4 |
| Forward-Left | -1 | -1 | -1 | 0 | 0 | 0 | 5 |
| Backward-Right | 1 | 1 | -1 | 2 | 2 | 0 | 6 |
| Backward-Left | -1 | 1 | -1 | 0 | 2 | 0 | 7 |

Movemet[3, 3, 3]

{

{

{ 5, 5, 5 },

{ 3, 3, 3 },

{ 7, 7, 7 }

},

{

{ 0, 0, 0 },

{ 9,-1, 8 },

{ 1, 1, 1 }

},

{

{ 4, 4, 4 },

{ 2, 2, 2 },

{ 6, 6, 6 }

}

};

anti_windup_beta :

  Start with 0.4–0.5 (good compromise).

  If you still see overshoot when coming off saturation → increase it a bit.

  If you see the control output “snap” abruptly after saturation → decrease it..