#include "main.h"

#include "mavlink/all/mavlink.h"

TIM_HandleTypeDef htim2;

UART_HandleTypeDef huart1;

DMA_HandleTypeDef hdma_usart1_tx;

DMA_HandleTypeDef hdma_usart1_rx;

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_DMA_Init(void);

static void MX_USART1_UART_Init(void);

static void MX_TIM2_Init(void);

#define MAVLINK_MSG_BUFFER_SIZE 512

#define MAVLINK_BUFFER_SIZE 512

#define MAV_SYSTEM_ID    1

#define MAV_COMPONENT_ID 1

#define MESSAGE_LEN 24

volatile bool uart1_tx_busy = false;

/*

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim) {

if(htim->Instance == TIM2)

{

send_mavlink_heartbeat();

HAL_Delay(100);

Send_Status_Text(1,1,"POWER OFF",3);

HAL_Delay(10);

Send_Status_Text(1,1,"POWER ON",3);

HAL_Delay(10);

Send_Status_Text(1,1,"Motors Enables",3);

HAL_Delay(10);

Send_Status_Text(1,1,"Motors Calibrations",3);

HAL_Delay(10);

Send_Status_Text(1,1,"ROV Move",3);

HAL_Delay(10);

Send_Status_Text(1,1,"Gripper opened",3);

HAL_Delay(10);

send_mavlink_heartbeat0();

HAL_Delay(10);

}

}*/

/*static uint8_t tx_buffer[MAVLINK_MAX_PACKET_LEN];

void Send_Status_Text(uint8_t system_id, uint8_t component_id, const char* text, uint8_t severity)

{

mavlink_message_t msg;

uint8_t id = 0;

uint8_t chunk_seq = 0;

uint16_t text_len = strlen(text);

for (uint16_t i = 0; i < text_len; i += 50)

{

char chunk[50] = {0};

strncpy(chunk, &text[i], 50);

if (strlen(chunk) == 0)

break;

mavlink_msg_statustext_pack(system_id, component_id, &msg, severity, chunk, id, chunk_seq);

uint16_t bytes_to_send = mavlink_msg_to_send_buffer(tx_buffer, &msg);

while (uart1_tx_busy);

uart1_tx_busy = true;

HAL_UART_Transmit_DMA(&huart1, tx_buffer, bytes_to_send);

if (strlen(chunk) < 50)

break;

chunk_seq++;

}

}*/

void send_mavlink_heartbeat(void) {

mavlink_message_t msg;

uint8_t buf[MAVLINK_MAX_PACKET_LEN];

mavlink_msg_heartbeat_pack(

MAV_SYSTEM_ID,

MAV_COMPONENT_ID,

&msg,

MAV_TYPE_QUADROTOR,

MAV_AUTOPILOT_GENERIC,

MAV_MODE_GUIDED_ARMED,

0,

MAV_STATE_ACTIVE

);

uint16_t len = mavlink_msg_to_send_buffer(buf, &msg);

HAL_UART_Transmit_DMA(&huart1, buf, len);

}

void send_mavlink_heartbeat0(void)

{

mavlink_message_t msg;

uint8_t buf[MAVLINK_MAX_PACKET_LEN];

mavlink_msg_heartbeat_pack(

MAV_SYSTEM_ID,

MAV_COMPONENT_ID,

&msg,

MAV_TYPE_SURFACE_BOAT,

MAV_AUTOPILOT_GENERIC,

MAV_MODE_MANUAL_ARMED,

0,

MAV_STATE_ACTIVE);

uint16_t len = mavlink_msg_to_send_buffer(buf, &msg);

HAL_UART_Transmit_DMA(&huart1, buf, len);

}/*

void send_mavlink_vfr_hud(float speed,float heading, float depth)

{

mavlink_message_t msg;

uint8_t buffer[MAVLINK_MAX_PACKET_LEN];

mavlink_msg_vfr_hud_pack(

1,

1,

&msg,

0,

speed,

heading,

0,

depth,

0

);

uint16_t len = mavlink_msg_to_send_buffer(buffer, &msg);

HAL_UART_Transmit_DMA(&huart1, buffer, len);

}

void send_attitude(float roll, float pitch, float yaw) {

mavlink_message_t msg;

uint8_t buffer[MAVLINK_MAX_PACKET_LEN];

mavlink_attitude_t attitude = {

.time_boot_ms = HAL_GetTick(),

.roll = roll,

.pitch = pitch,

.yaw = yaw,

.rollspeed = 0,

.pitchspeed = 0,

.yawspeed = 0

};

mavlink_msg_attitude_encode(1, MAV_COMP_ID_AUTOPILOT1, &msg, &attitude);

uint16_t len = mavlink_msg_to_send_buffer(buffer, &msg);

HAL_UART_Transmit_DMA(&huart1, buffer, len);

}

void send_Water_temp_pres(float pressure_hpa, float temp_celsius) {

mavlink_message_t msg;

uint8_t buffer[MAVLINK_MAX_PACKET_LEN];

mavlink_scaled_pressure_t msg_data;

msg_data.time_boot_ms = HAL_GetTick();

msg_data.press_abs = pressure_hpa;

//msg_data.press_diff = diff_pressure_hpa;

msg_data.temperature = (int16_t)(temp_celsius);

mavlink_msg_scaled_pressure_encode(1, MAV_COMP_ID_AUTOPILOT1, &msg, &msg_data);

uint16_t len = mavlink_msg_to_send_buffer(buffer, &msg);

HAL_UART_Transmit_DMA(&huart1, buffer, len);

}

void send_Tube_temp_pres(float pressure_hpa, float temp_celsius)

{

mavlink_message_t msg;

uint8_t buffer[MAVLINK_MAX_PACKET_LEN];

mavlink_scaled_pressure2_t msg_data;

msg_data.time_boot_ms = HAL_GetTick();

msg_data.press_abs = pressure_hpa;

//msg_data.press_diff = diff_pressure_hpa;

msg_data.temperature = (int16_t)(temp_celsius);

mavlink_msg_scaled_pressure2_encode(1, MAV_COMP_ID_AUTOPILOT1, &msg, &msg_data);

uint16_t len = mavlink_msg_to_send_buffer(buffer, &msg);

HAL_UART_Transmit_DMA(&huart1, buffer, len);

}

void func(int i)

{

switch(i)

{

case 0:

Send_Status_Text(1,1,"POWER ON",3);

break;

case 1:

Send_Status_Text(1,1,"Thrusters Enables",3);

break;

case 2:

Send_Status_Text(1,1,"Thrusters Calibrations",3);

break;

case 3:

Send_Status_Text(1,1,"ROV Ready",3);

break;

case 4:

Send_Status_Text(1,1,"ROV Move",3);

break;

case 5:

Send_Status_Text(1,1,"ROV Stop",3);

break;

case 6:

Send_Status_Text(1,1,"Gripper Opened",3);

break;

case 7:

Send_Status_Text(1,1,"Gripper Closed",3);

break;

case 8:

Send_Status_Text(1,1,"Thrusters Disables",3);

break;

case 9:

Send_Status_Text(1,1,"POWER OFF",3);

default:

break;

}

HAL_Delay(10);

}*/

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

{

if(htim->Instance == TIM2)

{

// send_mavlink_heartbeat();

}

}

uint8_t mavlink_rx_dma_buffer[MAVLINK_BUFFER_SIZE];

uint8_t mavlink_tx_buffer[MAVLINK_MSG_BUFFER_SIZE];

#define TX_QUEUE_SIZE 10

void Send_Command_Ack(uint8_t system_id, uint8_t component_id, uint16_t command, uint8_t result)

{

mavlink_message_t msg;

mavlink_msg_command_ack_pack

(

system_id,          // sender system ID

component_id,       // sender component ID

&msg,               // output message

command,            // command being acknowledged

result,             // result of command

0,                  // progress (optional, default 0)

0,                  // result_param2 (optional, default 0)

0,                  // target_system (optional, default 0)

0                   // target_component (optional, default 0)

);

static uint8_t tx_buffer[MAVLINK_MAX_PACKET_LEN];

uint16_t len = mavlink_msg_to_send_buffer(tx_buffer, &msg);

HAL_UART_Transmit_DMA(&huart1, tx_buffer, len);

}

void Send_Status_Text(uint8_t system_id, uint8_t component_id, const char* text, uint8_t severity)

{

mavlink_message_t msg;

uint8_t id = 0;

uint8_t chunk_seq = 0;

uint16_t text_len = strlen(text);

for (uint16_t i = 0; i < text_len; i += 50)

{

char chunk[50] = {0};

strncpy(chunk, &text[i], 50);

if (strlen(chunk) == 0)

break;

mavlink_msg_statustext_pack(system_id, component_id, &msg, severity, chunk, id, chunk_seq);

uint8_t buffer[MAVLINK_MAX_PACKET_LEN];

uint16_t bytes_to_send = mavlink_msg_to_send_buffer(buffer, &msg);

if (!uart1_tx_busy)

{

uart1_tx_busy = true;

HAL_UART_Transmit_DMA(&huart1, buffer, bytes_to_send);

HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);

}

if (strlen(chunk) < 50)

break;

chunk_seq++;

}

}

void Start_UART_DMA_Receive()

{

HAL_UARTEx_ReceiveToIdle_DMA(&huart1, mavlink_rx_dma_buffer, MAVLINK_BUFFER_SIZE);

}

void Process_Mavlink_Message(mavlink_message_t* msg)

{

switch (msg->msgid)

{

case MAVLINK_MSG_ID_COMMAND_LONG:

{

mavlink_command_long_t cmd;

mavlink_msg_command_long_decode(msg, &cmd);

if (cmd.command == MAV_CMD_DO_FLIGHTTERMINATION)

{

if(cmd.param1>0)

{

//HAL_GPIO_WritePin(GPIOA, GPIO_PIN_6, GPIO_PIN_RESET);

//HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

//Send_Status_Text(1,1,"Motors Power ON",4);

}

else

{

//HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET);

//HAL_GPIO_WritePin(GPIOA, GPIO_PIN_6, GPIO_PIN_SET);

//Send_Status_Text(1,1,"Motors Power OFF",4);

}

//Send_Command_Ack(1, 1, cmd.command, MAV_RESULT_ACCEPTED);

}

else if (cmd.command == MAV_CMD_COMPONENT_ARM_DISARM)

{

//if(cmd.param1>0)

//Send_Status_Text(1,1,"Motors Enabled",4);

//else

//Send_Status_Text(1,1,"Motors Disabled",4);

}

else if(cmd.command == MAV_CMD_DO_SET_SERVO)

{

//if(cmd.param1>0)

//Send_Status_Text(1,1,"Light ON",4);

//else

//Send_Status_Text(1,1,"Light OFF",4);

}

break;

}

case MAVLINK_MSG_ID_MANUAL_CONTROL:{

mavlink_manual_control_t cmd;

mavlink_msg_manual_control_decode(msg, &cmd);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_3, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_6, GPIO_PIN_RESET);

if(cmd.x>=7)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_3, GPIO_PIN_SET);

}

else if(cmd.x<=-7)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

}

if(cmd.y>=7)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_6, GPIO_PIN_SET);

}

else if(cmd.y<=-7)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, GPIO_PIN_SET);

}

break;

}

default:

break;

}

}

void HAL_UARTEx_RxEventCallback(UART_HandleTypeDef *huart, uint16_t Size)

{

if (huart->Instance == USART1)

{

mavlink_message_t local_msg;

mavlink_status_t local_status;

for (uint16_t i = 0; i < Size; i++)

{

if (mavlink_parse_char(MAVLINK_COMM_0, mavlink_rx_dma_buffer[i], &local_msg, &local_status))

{

Process_Mavlink_Message(&local_msg);

}

}

// HAL_StatusTypeDef status =

HAL_UARTEx_ReceiveToIdle_DMA(&huart1, mavlink_rx_dma_buffer, MAVLINK_BUFFER_SIZE);

/* if (status != HAL_OK)

{

//Send_Status_Message(1, 1, "DMA Restart Failed!", 4);

}*/

}

}

void HAL_UART_TxCpltCallback(UART_HandleTypeDef *huart)

{

if (huart->Instance == USART1)

{

uart1_tx_busy = false;

}

}

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_DMA_Init();

MX_USART1_UART_Init();

MX_TIM2_Init();

HAL_NVIC_SetPriority(TIM2_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(TIM2_IRQn);

// HAL_TIM_Base_Start_IT(&htim2);

Start_UART_DMA_Receive();

//int i=0;

while (1)

{

send_mavlink_heartbeat();

HAL_Delay(300);

send_mavlink_heartbeat0();

HAL_Delay(300);

/*send_Water_temp_pres(i,i);

HAL_Delay(10);

send_Tube_temp_pres(i+5,i+5);

HAL_Delay(10);

send_attitude(i,i,i);

HAL_Delay(10);

send_mavlink_vfr_hud(i,i,i);

HAL_Delay(10);

if(i%10==0)

func(i/10);

HAL_Delay(300);

i++;

if(i>99)

i=0;*/

}

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = 8;

RCC_OscInitStruct.PLL.PLLN = 84;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 4;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV2;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief TIM2 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM2_Init(void)

{

/* USER CODE BEGIN TIM2_Init 0 */

/* USER CODE END TIM2_Init 0 */

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

/* USER CODE BEGIN TIM2_Init 1 */

/* USER CODE END TIM2_Init 1 */

htim2.Instance = TIM2;

htim2.Init.Prescaler = 8399;

htim2.Init.CounterMode = TIM_COUNTERMODE_UP;

htim2.Init.Period = 9999;

htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim2.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim2) != HAL_OK)

{

Error_Handler();

}

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim2, &sClockSourceConfig) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim2, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM2_Init 2 */

/* USER CODE END TIM2_Init 2 */

}

/**

* @brief USART1 Initialization Function

* @param None

* @retval None

*/

static void MX_USART1_UART_Init(void)

{

/* USER CODE BEGIN USART1_Init 0 */

/* USER CODE END USART1_Init 0 */

/* USER CODE BEGIN USART1_Init 1 */

/* USER CODE END USART1_Init 1 */

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USART1_Init 2 */

/* USER CODE END USART1_Init 2 */

}

/**

* Enable DMA controller clock

*/

static void MX_DMA_Init(void)

{

/* DMA controller clock enable */

__HAL_RCC_DMA2_CLK_ENABLE();

/* DMA interrupt init */

/* DMA2_Stream2_IRQn interrupt configuration */

HAL_NVIC_SetPriority(DMA2_Stream2_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(DMA2_Stream2_IRQn);

/* DMA2_Stream7_IRQn interrupt configuration */

HAL_NVIC_SetPriority(DMA2_Stream7_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(DMA2_Stream7_IRQn);

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOA_CLK_ENABLE();

GPIO_InitTypeDef GPIO_InitStruct = {0};

GPIO_InitStruct.Pin = GPIO_PIN_5;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;    // Push-Pull Output

GPIO_InitStruct.Pull = GPIO_NOPULL;            // No pull-up/down

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;   // Low speed for LED

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_3;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_4;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_6;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */